SPE Paper Proposal — Abstract

Sorption Recovery of Lithium from High-Salinity Produced Water: How Brine Composition Sets Technical and Economic Limits

Authors: Nina A. Mkrtichian1, Mariia A. Mkrtichian2

Affiliations: 1Saint Petersburg State University; 2Empress Catherine II Saint Petersburg Mining University

Contact: st129824@student.spbu.ru; marimkr21@gmail.com

Suggested technical category: Water Management (produced water resource recovery) / Energy Transition (critical minerals from oilfield brines)

Keywords: produced water, oilfield brine, direct lithium extraction, sorption, layered double hydroxide, pretreatment, lithium carbonate, techno-economics, Eastern Siberia

Objectives/Scope   (58 words; SPE guideline 25–75)

Produced water is increasingly considered an unconventional lithium source, yet lithium concentration alone does not determine project feasibility. This study defines the combinations of lithium concentration, salinity, Mg/Li and Ca/Li ratios, annual water volume and sorbent service life under which a pretreatment–sorption–desorption–Li2CO3 precipitation flowsheet is technically robust and economically justified, focusing on calcium chloride brines of Eastern Siberia.

Methods, Procedures, Process   (97 words; SPE guideline 75–100)

Data published in 2019–2026 on produced water composition, pretreatment efficiency and the performance of manganese, titanium and lithium–aluminum sorbents in real field waters were systematized. For comparable water samples, Mg/Li and Ca/Li ratios and the stoichiometric reagent demand for complete softening (Na2CO3 for Ca2+, NaOH for Mg2+) were compared with the value of the contained lithium. A stage-by-stage lithium mass balance was compiled for three scenarios based on Russian field data, and a break-even lithium concentration was derived as a function of water volume, Li2CO3 price, overall recovery, capital cost and sorbent life, with ±30% sensitivity analysis.

Results, Observations, Conclusions   (179 words; SPE guideline 100–200)

Lithium content in produced water does not correlate with salinity and must be measured directly. For the Znamenskoye and Kovykta brines (Li ≈0.4 g/L, Ca 121–154 g/L, Mg/Li ≈72), reagents for complete softening cost 4.7–5.5 times more than the contained lithium (≈92 vs. ≈20 USD/m3), whereas for Na–Cl water of moderate hardness the ratio is only 0.22. Pretreatment should therefore be limited to removing oil, suspended solids, Fe, Mn and H2S, while lithium is separated from Ca and Mg by a lithium–aluminum layered double hydroxide sorbent (DGAL-Cl type) that cuts the alkaline-earth-to-lithium ratio 40–70-fold in one step and is desorbed with fresh water. Overall recovery is 0.44–0.78, with the main losses at the sorption and product stages. At 9,000 USD/t Li2CO3 (2025 average), the break-even lithium concentration is ≈270 mg/L at 1.5 million m3/year (≈26,000 bbl/d) and ≈170 mg/L at 10 million m3/year; at 20,000 USD/t (September 2026) it falls to 97 and 60 mg/L. Treated water from oil treatment units (30–70 mg/L Li) remains uneconomic, while deep brines (170–410 mg/L) produced at several million m3/year reach ≈4,700 USD/t.

Novel/Additive Information   (61 words; SPE guideline 25–75)

Unlike sorbent studies on synthetic solutions, the work links brine composition directly to flowsheet design and break-even limits. It shows that for calcium chloride brines the sorbent's main role is Li/Ca/Mg separation rather than concentration, identifies sorbent service life on real water as the key economic uncertainty, and proposes pilot-site criteria: ≥200 mg/L Li, ≥1.5–3 million m3/year and existing pressure-maintenance infrastructure.

Total: 395 words in four sections. Units: SI, with oilfield units in parentheses where helpful. No commercial or trade names are used; DGAL-Cl denotes the generic LiCl·2Al(OH)3 sorbent class.